PROMAX REMOTE

Remote Control System shall be the Pro Max™ Series as manufactured under the brand name Irritrol.

The remote system shall be compatible for use with any manufacturers’ 24 volt controller, have a compact transmitter with belt clip, Auto-up or Auto-down operation, independent control of master valve and/or pump, single or multi-station on control capability, timed station operation selectable from 1 to 60 minutes, communication and control of up to 999 individual receivers from a single transmitter, program ON/OFF capability when used with the RME Sentar, RME Eagle, or Evolution DX2 controllers and shall have a small, compact receiver for permanent internal mount in the controller, capability to detect and protect against field wiring short circuits and provide an audible alert, automatically reset circuit breakers and require no fuses, have a built-in safety default in its receiver to automatically turn station off after 60 minutes, provide audible transmitter and receiver tones to verify proper operation, include “power miser” circuitry in transmitter to eliminate down time for battery recharging and instead have a single, easily replaceable, widely available, lithium battery and shall provide audible alarm for low power alert.

Construction: The remote system shall have mechanisms to reduce interference and insure reliable communication and shall be of all-metal, water-and mud-resistant construction with shock absorbing bumpers and shall be available in a hard cover carrying case.

Electrical:

Input Power

• Receiver, 22-32 V ac, 50/60 Hz

• Universal adapter, 22-32 V ac, 50/60 Hz

• Transmitter, 6 volt lithium battery (user replaceable) Output Power

• Receiver/Universal adapter, 24 V ac, 1.5 amps maximum total output 	   (36VA), 1 amp per station or mastervalve/pump