JUNIOR DC SERIES

The automatic controller shall be Junior DC series with 1 and 4 station models as manufactured under the Irritrol brand name and installed in accordance with local codes and manufacturer’s instructions.

Operation: Controller shall be battery-operated, requiring no A.C. power, and shall have automatic, semiautomatic and manual operational modes. Two mounting devices shall be provided with each controller: a solenoid bracket for mounting controller directly on the valve solenoid and a wall-mount bracket for controller attachment to a flat surface. Because of its valve-mount option, controller shall be sufficiently waterproof to provide reliable operation down in a valve box, despite occasional flooding. A DC, latching solenoids shall be required on any valve to be operated by controller. Controller shall have two programs with three start times each and a program stacking feature to prevent operational overlap. Watering time per station shall be in a range from 0 to 155 minutes settable in 1-minute increments. Controller shall provide two types of watering day selections; 7-day “select day” watering or watering day intervals for 1 to 14 days. Controller shall have an OFF setting for system shut down and shall also operate with an optional, normally-closed, wired rain sensor. Controller shall have a water budgeting feature allowing program watering time adjustments, in 10% increments, from 10% of set watering time up to 200%.

Construction: Controller shall be constructed from a durable polymer with a 0-ring-sealed battery compartment and a flip-down cover to protect its liquid crystal display and programming buttons.

Electrical: Controller shall require a user-supplied, 9-volt, alkaline battery for power and its output shall be via DC pulse requiring a DC, latching solenoid on the valve. Maximum wire length between controller and the valve shall be from 660’ to 960’ depending upon wire size. Controller shall be cULus listed.